Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

SECTION 072500 - WEATHER BARRIERS

GENERAL

SECTION INCLUDES

- Note to Specifier: “Building wrap” refers to a polymeric rolled membrane that resists bulk moisture penetration while allowing moisture vapor transmission.

A. Water-resistive barrier (GRIP-RITE® Housewrap-LWE)

Seam tape and related accessories

RELATED SECTIONS

Section 06 16 00 – Sheathing

Section 07 21 00 – Thermal Insulation

Section 07 60 00 – Flashing and Sheet Metal

Section 07 26 00 – Vapor Retarders

REFERENCE STANDARDS

ASTM E96 – Water Vapor Transmission of Materials

ASTM D882 – Tensile Properties of Thin Plastic Sheeting

ASTM D779 – Water Resistance of Sheet Materials

ASTM E84 – Surface Burning Characteristics of Building Materials

ASTM D5261 – Mass per Unit Area of Geotextiles

ICC-ES AC38 – Acceptance Criteria for Water-Resistive Barriers

SUBMITTALS

Product Data: Manufacturer’s current technical literature.

Samples: Minimum 8-1/2 inch by 11 inch sample of membrane.

Installation Instructions: Manufacturer’s written installation guidelines.

Warranty Documentation.

QUALITY ASSURANCE

Deliver materials in original, unopened packaging with labels intact.

Store materials in a dry location, protected from moisture and direct sunlight.

Protect materials from damage during handling and installation.

DELIVERY, STORAGE, AND HANDLING

Deliver materials in original, unopened packaging with labels intact.

Store materials in a dry location, protected from moisture and direct sunlight.

Protect materials from damage during handling and installation.

ENVIRONMENTAL REQUIREMENTS

Do not install over wet, frozen, or frost-covered substrates.

Allow wet surfaces to dry before installation of exterior cladding.

PRODUCTS

WEATHER BARRIERS

- GRIP-RITE®

- PrimeSource Building Products, Inc.

- Website: www.grip-rite.com

MATERIALS

- A. GRIP-RITE® Housewrap-LWE

- Description: Lightweight economy-grade weather-resistive barrier designed to reduce air infiltration and resist bulk water penetration while allowing moisture vapor transmission.

- Material: Cross-woven polyolefin fabric with polyolefin coating layer; micro-perforated for enhanced vapor transmission.

- Water Vapor Permeance: Approximately 10 perms (ASTM E96).

- Water Resistance: Meets ASTM D779 requirements.

- Tensile Strength: Meets ASTM D882 requirements.

- Surface Burning Characteristics: Class A (ASTM E84).

- Compliance: Meets ICC-ES AC38 requirements for water-resistive barriers when installed per manufacturer instructions.

ACCESSORIES

Fasteners: Non-corrosive roofing nails, plastic cap fasteners, or non-corrosive staples with minimum 1-inch crown. Fasteners shall penetrate sheathing or framing a minimum of 1/2 inch.

Seam Tape: Compatible sheathing or housewrap tape recommended by manufacturer for sealing overlaps and minor repairs.

WARRANTY

- Manufacturer’s standard limited warranty as published at time of specification.

EXECUTION

EXAMINATION

Examine jobsite and structure prior to installation.

Notify architect if conditions adversely affect installation or do not comply with manufacturer instructions.

INSTALLATION OF WEATHER BARRIERS

Install building wrap over exterior sheathing in accordance with manufacturer’s current installation recommendations.

Begin installation 12 to 24 inches from a corner with printed side facing outward. Secure starting vertical edge with fasteners spaced 6 inches on center.

Install horizontally from bottom of wall upward in shingle fashion. Extend lower edge 2 to 3 inches over sill plate interface.

Fasten 8 inches on center along plates and 24-inch intervals at framing members. Increase fastening frequency around openings.

Overlap subsequent courses minimum 2 inches horizontally and 6 inches vertically.

Tape seams and integrate with flashing at windows, doors, and penetrations to maintain continuous drainage plane.

PROTECTION

Cover with exterior cladding within manufacturer’s maximum allowable UV exposure period.

Protect installed building wrap from damage during construction.

END OF SECTION 072500