Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

SECTION 072500 - WEATHER BARRIERS

GENERAL

SECTION INCLUDES

Mechanically-installed weather-resistive barrier.

GRIP-RITE® WeatherResistor HP.

Sheathing/Seam Tape (GRIP-RITE® House Wrap Tape)

- Note to Specifier: “Building wrap” refers to a polymeric rolled membrane designed to resist bulk water penetration while allowing water vapor transmission.

RELATED SECTIONS

Section 06000 – Sheathing for exterior wall sheathing

Section 07100 – Dampproofing and Waterproofing

Section 07210 – Building Insulation

Section 07260 – Vapor Retarders

REFERENCE STANDARDS

- ASTM E96 – Water Vapor Transmission of Materials

- ASTM D5034 – Breaking Strength and Elongation (Grab Test)

- ASTM D779 – Water Resistance (Dry Indicator Method)

- ASTM E84 – Surface Burning Characteristics of Building Materials

- ASTM D5261 – Mass per Unit Area of Geotextiles

- ASTM E2178 – Air Permeance of Building Materials

- AATCC 127 – Hydrostatic Pressure Test

- ICC-ES AC38 – Acceptance Criteria for Water-Resistive Barriers

SUBMITTALS

Product Data: Manufacturer’s technical data sheets.

Samples: Minimum 8-1/2 by 11 inch sample of membrane.

Installation Instructions: Current manufacturer installation guidelines.

Warranty Documentation.

QUALITY ASSURANCE

Installer shall have experience with similar weather barrier assemblies.

Installation shall comply with manufacturer’s written instructions.

Product shall meet ICC-ES AC38 requirements.

DELIVERY, STORAGE, AND HANDLING

Deliver materials in original, unopened packaging with labels intact.

Store materials as recommended by manufacturer.

Protect from damage during handling and installation.

ENVIRONMENTAL REQUIRMENTS

Do not install over wet or frost-covered substrates.

Ensure substrate is clean, dry, and free of damage prior to installation.

PRODUCTS

MANUFACTURER

- GRIP-RITE®

- PrimeSource Building Products, Inc.

- 1321 Greenway Drive, Irving, TX 75038-2504

- Phone: 972-999-8500

- Website: www.grip-rite.com

MATERIALS

GRIP-RITE® WeatherResistor HP

Description: High-performance, non-woven, non-perforated housewrap designed to function as a water-resistive barrier and air barrier while allowing vapor permeability.

Material: Multi-layer polypropylene construction; non-perforated.

Typical Performance Characteristics

Basis Weight: 2.3 oz/yd² (ASTM D5261)

Water Vapor Permeance: 50 perms (ASTM E96)

Air Permeance: 0.02 L/s/m² (ASTM E2178)

Water Penetration Resistance (Hydrostatic Head): 400 cm (AATCC 127)

Tensile Strength (MD/CD): 65/45 lb/in (ASTM D5034)

Tear Strength (MD/CD): 8/10 lb

Surface Burning Characteristics: Flame Spread Index 25; Smoke Developed 450 (ASTM E84)

UV Exposure Limit: 180 days

Compliance: Meets ICC-ES AC38 requirements for water-resistive barriers.

ACCESSORIES

Fasteners: Corrosion-resistant roofing nails, plastic cap nails, or cap staples penetrating framing or sheathing minimum 1/2 inch.

Seam Tape: Compatible manufacturer-recommended tape for sealing seams and penetrations.

Flashing and Sealants: Compatible flashing tapes and sealants meeting ASTM C920 where applicable.

WARRANTY

- Manufacturer’s standard limited warranty as published at time of project specification.

EXECUTION

EXAMINATION

Verify sheathing is properly installed and free of damage.

Do not proceed with installation until unsatisfactory conditions are corrected.

INSTALLATION

Install WeatherResistor HP over exterior sheathing with printed side facing outward.

Install horizontally, beginning at base of wall, working upward in shingle fashion.

Overlap horizontal seams minimum 2 inches; vertical seams minimum 6 inches.

Fasten in accordance with manufacturer’s instructions and applicable code requirements.

Seal all seams, penetrations, and openings with compatible tape or flashing.

Integrate with window and door flashing systems to maintain continuous drainage plane.

PROTECTION

Cover with exterior cladding within manufacturer’s maximum UV exposure limit (180 days).

Protect installed material from damage until permanent cladding is installed.

END OF SECTION 072500